TECHNICAL SPECIFICATION

(for Urban, Rural, and Sub Branches)

All the Functionalities are Mandatory and should be available in the offered solution as a standard product. In case, any of these are not offered as standard products, the bid may be made non-responsive.

Technical requirements summary (total for 3 Urban, 3 Rural, and 4 Sub-branches):

| A. | BOQ Compliance for the LAN Infrastructure Accessories of the Urban, Rural, and Sub Branches |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL | Particulars | Urban (Qty.) | Rural (Qty.) | Sub (Qty.) | Units |
| 1 | Wall-mount 12U Server/Network Cabinet | 3 | - | - | Set (s) |
| 2 | Wall-mount 9U Server/Network Cabinet | - | 3 | 4 | Set (s) |
| 3 | Wire Manager, 1U 19” Horizontal | 6 | 3 | 4 | Nos |
| 4 | CAT6 UTP Modular Patch Panel 24-Ports with Modular/Keystone | 6 | 3 | 4 | Nos |
| 5 | Dual Port Square Type Faceplate | 120 | 99 | 80 | Nos |
| 6 | CAT6 UTP Modular Jack | 120 | 99 | 80 | Nos |
| 7 | CAT6 RJ45-UTP 4-Pair Patchcord 1M | 120 | 99 | 80 | Nos |
| 8 | CAT6 RJ45-UTP 4-Pair Patchcord 2M | 120 | 99 | 80 | Nos |
| 9 | CAT6 UTP RJ45 Metal Shielded Connector | 30 | 30 | 40 | Pc (s) |
| 10 | Wall-mount Rack, and Node installation with necessary accessories (PVC Pipe, Channel, Flexible pipe, Royal Plug, Angles, Bolt, tape, screw, etc.) | Mentioned by the bidder | Mentioned by the bidder | Mentioned by the bidder | Nos |
| 11 | UTP Node Installation, Dressing, and Identification Marking (Tagging) of network accessories at the ISO standard. | 120 | 99 | 80 | Nos |

| B. | BOQ Compliance for the Physical Access Security Accessories of the Urban, Rural, and Sub Branches |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL | Particulars | Urban (Qty.) | Rural (Qty.) | Sub (Qty.) | Units |
| 1 | ZKTeco F18 Fingerprint Standalone Access Control Machine | 3 | 3 | 4 | Set (s) |
| 2 | EM Lock LM-280 | 3 | 3 | 4 | Nos |
| 3 | U/ZL Bracket | 3 | 3 | 4 | Nos |
| 4 | Wireless Push Button | 3 | 3 | 4 | Nos |
| 5 | Exit Button | 3 | 3 | 4 | Nos |
| 6 | Accessories | 3 | 3 | 4 | Nos |
| 7 | Installation Device | 3 | 3 | 4 | Nos |

| C. | BOQ Compliance for the Attendance System of the Urban, Rural, and Sub Branches |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL | Particulars | Urban (Qty.) | Rural (Qty.) | Sub (Qty.) | Units |
| 1 | ZKTeco F18 Time Attendance | 3 | 3 | 4 | Nos |
| 2 | Installation of Access control and attendance Device, Configuration, Data Entry, and finger Entry. | 3 | 3 | 4 | Nos |

| D. | BOQ Compliance for the Generic SIP Phones of the Urban, Rural, Sub-Branches, and Head Office |  |  |
| --- | --- | --- | --- |
| SL | Particulars | Qty. | Units |
| 1 | Generic SIP Phone (Model: Grandstream GRP2601P/GRP2602P) | 48 | Nos |

Detail TECHNICAL SPECIFICATIONS of Products for 3 Urban, 3 Rural, and 4 Sub Branches

| S/L | Items |  | Descriptions |  | Urban (Qty.) |  |  | Rural (Qty.) |  | Sub (Qty.) |  | Unit (s) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. | Branch LAN Infrastructure Accessories |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12U Wall Mount |  | Brand: Cote |  | 3 |  | - |  |  | - |  | Set (s) |  |
|  | Server/Network Cabinet |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Malaysia |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Size: 12U |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dimensions: (600 x 450) mm |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: 2 x 4-inch Fan (Low Noise) on Vented Top Brush Panel. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Front Door –Glass with Handle Lock. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Side panels with round lock: 2PCS |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 x Rear panel with hanging plate |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 x Power Distribution Unit (6-Way Universal PDU) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20 Sets of Philip screws and Nuts. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Brush Panel on top and bottom cable entry. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Maximum Load Capacity (KG): Industry Standard. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Black |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
| 2 | 9U Wall Mount Server/Network Cabinet |  | Brand: Cote |  | - |  | 3 |  |  | 4 |  | Set (s) |  |
|  |  |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Malaysia |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Size: 9U |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dimensions: (600 x 450) mm |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: 2 x 4-inch Fan (Low Noise) on Vented Top Brush Panel. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Front Door –Glass with Handle Lock. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Side panels with round lock: 2PCS |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 x Rear panel with hanging plate |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1 x Power Distribution Unit (6-Way Universal PDU) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20 Sets of Philip screws and Nuts. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Brush Panel on top and bottom cable entry. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Maximum Load Capacity (KG): Industry Standard. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Black |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Brand: Cote |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
| 3 | Wire Manager |  | Brand: Rosenberger |  | 6 |  | 3 |  |  | 4 |  | Nos |  |
|  |  |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Type: 1U, Horizontal |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dimensions: Mention by bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: Horizontal, Special construction ensures patch cord bending to guarantee transmission performance, Standard 19”, widely used industry standard |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Material: Steel |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Black |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
| 4 | Cat6 UTP Modular Patch |  | Brand: Rosenberger |  | 6 |  | 3 |  |  | 4 |  | Nos |  |
|  | Panel, 24-port with |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  | Modular/Keystone |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Category: Cat6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Capacity: 24 Ports |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Size: 1U |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: Standard 19” widely used industry standard; each keystone can be individually disassembled. Plenty of tag space. Cat6 24 ports patch panel. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Footprint/Type: MDVO-Style |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Front Connection: Lead Frame- Copper Allow with 50u inch Gold over Nickel. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Product Structure: Individual Modular/keystone system. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Panel Front Connection- Flush |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Termination Area: Rear |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Modular Jack: Cat6 UTP |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Included Parts: Modular- 1 T-Bar, Panel- 4 Screws (10x32); 4 Screws (12x24); 1 Printable Label-flex Half-Sheet; 1 Rear Cable Management Bar, 1 Installation Guide. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Safety Listing: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
| 5 | Dual port Square Type |  | Brand: Rosenberger |  | 120 |  | 99 |  |  | 80 |  | Nos |  |
|  | Face Plate |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mentioned by bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Material: PC |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Size: (H x W x D) 86 x 86 x 10 mm |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Port No: 2 with shutter-type dust cover |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Inner Spring: Alloy Spring |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Compatible Jack: Support UTP, STP; CAT5e, CAT6, CAT6A |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: Anti Dust Protection |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: White |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
| 6 | Cat6 UTP Modular Jack |  | Brand: Rosenberger |  | 120 |  | 99 |  |  | 80 |  | Nos |  |
|  |  |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mention by bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Type: CAT6 UTP 90-Degree Toolless |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: 1.5 Amp max current rating, 150V voltage rating, 20m-ohm contact resistance, 500M-ohm insulation resistance, 1000V AC RMS, 60Hz 1min dielectric withstand voltage. Compatible with CAT5e, CAT6, and CAT6A copper cable. Shielded or unshielded option CM, CMR, CMP, and LSZH series fire retardant. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Method of Shielding: CAT-6 Unshielded |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Working Temperature: -25~60°C |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Wiring Scheme: T568A/B |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Included Parts: 1 T-Bar |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Footprint/Type: Key-Connect |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Plug/Connector Retention: 50.042 N |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Warranty: Mention by the bidder |  |  |  |  |  |  |  |  |  |  |
| 7 | Cat6 RJ45-RJ45 UTP |  | Brand: Rosenberger |  | 120 |  | 99 |  |  | 80 |  | Nos |  |
|  | 4-pair patchcord, 1M |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Length: 1 Meter |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Material: Bare copper (99.99%) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Method of Shielding: U/UTP (Unshielded) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jacket: CM (PVC) and LSZH |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insulation: HDPE |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Connector-Type: RJ45 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Red/Blue/Yellow |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Plug Housing: PC—Polycarbonate, UL 94 V-2 rated |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Contact Blade: Phosphor bronze plated with 50u” gold over 100u” nickel undercoat |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Current Rating: 1.5 A maximum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insulation Resistance: 100 MΩ minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dielectric Strength: 1000 VAC (RMS) minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Voltage Rating: 72 VDC minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insertion Life: 750 mating cycles minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Temperature Range: -10°C to 60°C |  |  |  |  |  |  |  |  |  |  |
| 8 | Cat6 RJ45-RJ45 UTP |  | Brand: Rosenberger |  | 120 |  | 99 |  |  | 80 |  | Nos |  |
|  | 4-pair patchcord, 2M |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Germany/USA |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Length: 2 meters |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Material: Bare copper (99.99%) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Method of Shielding: U/UTP (Unshielded) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jacket: CM (PVC) and LSZH |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insulation: HDPE |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Connector-Type: RJ45 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Red/Blue/Yellow |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Plug Housing: PC—Polycarbonate, UL 94 V-2 rated |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Contact Blade: Phosphor bronze plated with 50u” gold over 100u” nickel undercoat |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Current Rating: 1.5 A maximum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insulation Resistance: 100 MΩ minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dielectric Strength: 1000 VAC (RMS) minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Voltage Rating: 72 VDC minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Insertion Life: 750 mating cycles minimum |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Temperature Range: -10°C to 60°C |  |  |  |  |  |  |  |  |  |  |
| 9 | Cat6 UTP RJ45 Shielded |  | Brand: Mentioned by the bidder |  | 30 |  | 30 |  |  | 40 |  | Pc (s) |  |
|  | Connector |  | Model: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part Number: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Country of Origin: Mentioned by the bidder |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Cable Category: CAT-6 UTP |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Color: Silver / Grey |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Material: Metal |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Data Transfer Rate: 1000 Megabits per second |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Features: Cat6 RJ45 Connector LAN Connector. Aluminum shield, metal shield with load bar, and ethernet connector. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Operating temperature: -40°c to 80°c |  |  |  |  |  |  |  |  |  |  |
| 10 | Wall-mount Rack, and Node installation with necessary accessories (PVC Pipe, Channel, Flexible pipe, Royal Plug, Angles, Bolt, tape, screw, etc.) |  | Mentioned by the bidder |  | Mentioned by the bidder |  | Mentioned by the bidder |  |  | Mentioned by the bidder |  | Nodes |  |
| 11 | UTP Node Installation, Dressing, and Identification Marking (Tagging) of network accessories at the ISO standard. |  | Mentioned by the bidder |  | 120 |  | 99 |  |  | 80 |  | Nodes |  |
| S/L | Items |  | Descriptions |  | Urban (Qty.) | Rural (Qty.) |  |  | Sub (Qty.) |  |  | Unit (s) |  |
| B. | Physical Access Security Accessories |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ZKTeco F18 Fingerprint Standalone Access Control Machine |  | Features: • Model: ZK F18 • Less than 1-second user recognition • Stores 3000 fingerprint templates and 30,000 transactions • Reads fingerprint and/or RFID cards. • Built-in Serial and Ethernet ports |  | 3 | 3 |  |  | 4 |  |  | Nos |  |
| 2 | EM Lock LM-280 |  | Brand: ZKTeco; Made in China or Equivalent |  | 3 | 3 |  |  | 4 |  |  |  |  |
| 3 | U/ZL Bracket |  | Brand: ZKTeco; Made in China or Equivalent |  | 3 | 3 |  |  | 4 |  |  |  |  |
| 4 | Wireless Push Button |  | Good Quality; Made in China or Equivalent |  | 3 | 3 |  |  | 4 |  |  |  |  |
| 5 | Exit Button |  | Model: ABK 800B; Brand: YLI; Made in China or Equivalent |  | 3 | 3 |  |  | 4 |  |  |  |  |
| 6 | Accessories |  | Wiring cable, Power Cable, PVC, Flexible pipe, channel, cable clip, tape, screw &, etc. (as required) |  | 3 | 3 |  |  | 4 |  |  |  |  |
| 7 | Installation Device |  | Mentioned by the bidder |  | 3 | 3 |  |  | 4 |  |  | Nodes |  |
| S/L | Items |  | Descriptions |  | Urban (Qty.) | Rural (Qty.) |  |  | Sub (Qty.) |  |  | Unit (s) |  |
| C. | Attendance System |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | ZKTeco F18 Time Attendance |  | Features: • Model: ZK F18 • Less than 1-second user recognition • Stores 3000 fingerprint templates and 30,000 transactions • Reads fingerprint and/or RFID cards. • Built-in Serial and Ethernet ports |  | 3 | 3 |  |  | 4 |  |  | Nos |  |
| 2 | Installation of Access control and attendance Device, Configuration, Data Entry, and finger Entry. |  | Mentioned by the bidder |  | 3 | 3 |  |  | 4 |  |  | Nodes |  |
| S/L |  | Particulars |  | Descriptions of the particulars and features |  |  |  |  |  | Qty. | Unit (s) |  |  |
| D. |  | Generic SIP Phones |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Brand |  | Grandstream |  |  |  |  |  | 48 | Nos |  |  |
| 2 |  | Model |  | GRP2601P/GRP2602P |  |  |  |  |  |  |  |  |  |
| 3 |  | Country of Origin |  | To be mentioned by the bidder |  |  |  |  |  |  |  |  |  |
| 4 |  | General Features |  | Must have a minimum of 2 SIP lines |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Must be HD Voice quality |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Must be PoE enabled and optional external power supply as well. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2.4/2.8-inch color display/screen. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Must have the option of Handset/Hands-free/Headset Mode. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Including Desktop Stand. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Color: Black |  |  |  |  |  |  |  |  |  |
| 5 |  | Phone Features |  | Must have local phonebook |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Minimum of 600 entries of Call logs (in/out/missed) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Must have Black/White list Call Filter facility |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Voice Message Waiting Indication (VMWI) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Screen-saver |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Programmable DSS/Soft keys |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Network time synchronization |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Support Plantronics wireless headset |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Support Jabra wireless headset |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Support Call Recording |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Action URL / Active URI |  |  |  |  |  |  |  |  |  |
| 6 |  | Call Features |  | Must have the function of Call Out/Answer/reject |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Must have the option of Mute/Unmute (Microphone) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Call Hold/Resume |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Call Waiting, Intercom |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Call Forward (Always/Busy/No Answer) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Call Transfer (Attended/Unattended) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Call Parking/Pick-up |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Caller ID Display, Speed Dial |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Anonymous Call (Hide Caller ID) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Redial, Do not disturb, Auto-Answering |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Voice Message, and 3-way Conference |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Hot Line, Hot Desking |  |  |  |  |  |  |  |  |  |
| 7 |  | Network Features |  | Physical: 10/100 Mbps Ethernet, Dual bridge port for PC bypass |  |  |  |  |  |  |  |  |  |
|  |  |  |  | RJ9 Port x2: Handset x1, Head |  |  |  |  |  |  |  |  |  |
|  |  |  |  | RJ45 Port x2: Network x1, PC x1 (Bridged to Network) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | IP Mode: IPv4/IPv6 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | IP Configuration: Static IP/DHCP/PPPoE |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Network Access Control: 802.1x |  |  |  |  |  |  |  |  |  |
|  |  |  |  | VPN: L2TP/OpenVPN |  |  |  |  |  |  |  |  |  |
|  |  |  |  | VLAN, LLDP, QoS |  |  |  |  |  |  |  |  |  |
|  |  |  |  | RTCP-XR (RFC3611), VQ-RTCPXR (RFC6035) |  |  |  |  |  |  |  |  |  |
| 8 |  | Protocol Features |  | SIP RFC3261, TCP/IP/UDP, RTP/RTCP, RTCP-XR, HTTP/HTTPS, ARP, ICMP, DNS (A record, SRV, NAPTR), DHCP, PPPoE, SSH, TFTP, NTP, STUN, SIMPLE, LLDP, LDAP, TR069, SNMP, 802.1x, TLS, SRTP, IPv6 |  |  |  |  |  |  |  |  |  |
| 9 |  | Management Features |  | Must have Auto-provisioning via FTP/TFTP/HTTP/HTTPS/DHCP OPT66/SIP PNP/TR-069 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Web Management Portal |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Web-based Packet Dump |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Configuration Export/Import |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Phonebook Import/Export |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Firmware Upgrade |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Syslog |  |  |  |  |  |  |  |  |  |
| 10 |  | Audio Features |  | HD Voice Microphone/Speaker |  |  |  |  |  |  |  |  |  |
|  |  |  |  | HAC Handset |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Supported Codecs: G.722, G.711, G.723, G.726, G.729 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Full-Duplex Acoustic Echo Canceller (AEC) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Voice Activity Detection (VAD)/Comfort Noise Generation (CNG)/Background Noise Estimation (BNE)/Noise Reduction (NR) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Packet Loss Concealment (PLC) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dynamic Adaptive Jitter Buffer |  |  |  |  |  |  |  |  |  |
| 11 |  | Power and Environment |  | DC Power Input: 5V/0.6A |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Power Consumption: Idle 0.9~1.65W, Peak 1.8~2.95W |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Working Temperature: 0~45℃ |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Working Humidity: 10~95% |  |  |  |  |  |  |  |  |  |
| 12 |  | Warranty & support service |  | A minimum of one (1) year warranty. |  |  |  |  |  |  |  |  |  |